Donning (putting on) PPE

| 1 | PERFORM HAND HYGIENE - Wash hands or use an alcohol-based hand rub |  |
| --- | --- | --- |
| 2 | PUT ON GOWN - Fully cover torso from neck to knees, arms to end of wrists and back - Fasten at the back and neck |  |
| 3 | MASK OR RESPIRATOR - Coloured side is always placed to the outside - Secure ties or elastic bands at middle of head and neck - Fit flexible/metal band to nose bridge - Fit snug to face an below chin |  |
| IMPORTANT- IF USING A P2/N95 MASK A FIT CHECK MUST BE PERFORMED EVERY TIME |  |  |
| 4 | GOGGLES OR FACE SHIELD - Place over face and eyes and adjust to fit NOTE: prescription glasses are not appropriate to use as PPE, goggles must be placed over the prescription glasses. |  |
| 5 | GLOVES - Extend to cover wrist/cuff of isolation gown. - Ensure no gaps are present |  |

How to FIT CHECK a P2/N95 mask

- The wearer should forcefully inhale and exhale several times.

- The mask should collapse slightly upon inhaling and expand upon exhaling.

- The wearer should not feel any air leaking between face and the mask.

This is the sign of a good facial fit and a successful User Fit Check.

Doffing (taking off) PPE

| 1 | GOWN AND GLOVES - Grasp the gown in the front - Pull away from your body so that the ties break, touching outside of gown only with gloved hands - As you are removing the gown, peel off your gloves at the same time, only touching the inside of the gloves and gown with your bare hands. - Whilst removing the gown, fold or roll the gown inside out into a bundle. - Place the gown and glove bundle into general waste bin |  |
| --- | --- | --- |
| 2 | PERFORM HAND HYGIENE - Wash hands or use an alcohol-based hand rub |  |
| 3 | GOGGLES OR FACESHIELD - Remove goggles or face shield from the back by lifting side arms / head band and without touching the front of the goggles or face shield. - If disposable, discard into general waste. - If reusable place into designated recepticle for reprocessing |  |
| 4 | MASK OR RESPIRATOR - Grasp bottom ties or elastics of themask/respirator, then the ones at the top and remove without touching the front of the mask. - Discard into general waste |  |
| 5 | PERFORM HAND HYGIENE - Immediately after removing all PPE, wash hands or use an alcohol-based hand rub |  |

How to put on a P2/N95 mask

| - Separate the edges of the mask to fully open it. | - Slightly bend the nose wire to form a gentle curve. | - Hold the mask upside down to expose the two headbands. | - Using your index fingers and thumbs, separate the two headbands. | - While holding the headbands with your index fingers and thumbs, cup the mask under your chin. |
| --- | --- | --- | --- | --- |
| - Pull the headbands over your head. | - Release the lower headband from your thumbs and position it at the base of your neck. | - Position the remaining headband on the crown of your head. | - Conform the nosepiece across the bridge of your nose by firmly pressing down with your fingers. | - Continue to adjust the mask and secure the edges until you feel you have achieved a good facial fit. Now perform a Fit Check. |